AAROGYA JAL (PHASE 1): MULTI-TIER SENSOR SPECIFICATIONS & 3-QUOTE FINANCIAL ARCHITECTURE

Engineering Specifications, Limnological Pathogen-Proxy Physics, and Comprehensive 3-Tier Budget Options (Bare-Minimum vs. Value-Engineered Pilot vs. Full Municipal Enterprise) for Fateh Sagar Lake, Udaipur

| Project Site Fateh Sagar Lake, Udaipur | Deployment Stage Phase 1 Static Buoy | Target Stakeholders UMC, Smart City, iStart | Document Version Rev 3.5 (Three-Tier Ready) |
| --- | --- | --- | --- |
| Lead Engineering Manan Ramani & Team | Quote 1 (Bare-Minimum) ₹35,500 CapEx | Quote 2 (Pilot Sweet Spot) ₹71,500 CapEx | Quote 3 (Full Municipal) ₹3,15,000 CapEx |

# 1. Executive Financial Summary: The 3 Budget Quotes at a Glance

📌 Strategic Budget Sizing Rationale Depending on funding availability (Student Hackathon vs. BITS/iStart Grant vs. Municipal Smart City Tender), the hardware architecture can scale across three distinct cost tiers without discarding previously developed firmware or cloud codebases. • Quote 1 (₹35,500): Focuses on the 4-sensor sewage surrogate core for immediate proof-of-concept. • Quote 2 (₹71,500): Recommended sweet spot for a 3-6 month field pilot, adding DO, level, rain, and power autonomy. • Quote 3 (₹3,15,000): Municipal enterprise standard with autonomous motorized wipers, NABL certifications, and 5-year station life.

| System Dimension | Quote 1: Bare-Minimum Core | Quote 2: Value-Engineered Pilot | Quote 3: Municipal Full-Spec |
| --- | --- | --- | --- |
| Total Initial CapEx | ₹35,500 | ₹71,500 (Recommended) | ₹3,15,000 |
| Target Milestone | Student Hackathon / Bench Demo | BITS Pitch / iStart / 6-Mo Pilot | Smart City RFP / Permanent Station |
| Sensors Deployed | 4 Core: EC, Turbidity, pH, Temp | 7 Probes: DO, Turbidity, EC, pH, Temp, Level, Rain | 8 Probes: Wiped DO, Wiped Turbidity, EC, pH, Temp, Level, Rain, Baro |
| Anti-Biofouling System | Manual weekly wipe + Copper mesh | Manual fortnightly wipe + Copper shroud | Motorized Silicone Wipers (every 3h) + CuNi cage |
| Telemetry & Compute | ESP32 + 4G LTE-Cat1 Modem | ESP32-S3 + 4G LTE + SD Log + RTC | ESP32-S3 Industrial + Quectel 4G + Isolators |
| Power Harvesting | 20W Solar Panel + 12V 7Ah SLA | 25W Mono Panel + 12V 12Ah Battery | 50W Mono Panel + 12V 24Ah LiFePO4 (7-day autonomy) |
| Quality Assurance & Lab | College bio lab petri plates | College lab loaner meter + MacConkey | Apera Handheld Meter + Aquagenx CBT + 26 NABL Panels |

# 2. Disease Surveillance Framework: The Physical-Chemical Proxy Model

Direct, reagentless, electronic immersion probes for bacteria (E. coli, Vibrio cholerae, Salmonella typhi, Rotavirus) DO NOT EXIST globally for autonomous lake buoys due to instant membrane biofouling. Aarogya Jal monitors waterborne contamination and public health disease risk through real-time multi-parametric physical-chemical surrogates:

Sewage & Enteric Pathogen Influx (E. coli, Cholera, Typhoid, Gastroenteritis, Hepatitis A): Sewage contains concentrated urine electrolytes (Na+, Cl-, K+), suspended fecal particles, and organic matter. This triggers a simultaneous Electrical Conductivity (EC) spike (+50% to +150%), Optical Turbidity spike (>25 NTU), and Dissolved Oxygen (DO) crash (<4.0 mg/L) within 15 minutes of ingress.

Cyanobacterial Toxic Algal Blooms (Microcystins & Anatoxins): Photosynthetic CO2 consumption causes midday DO supersaturation (>120% to 160%) and alkaline pH spikes (>8.8), followed by lethal nighttime hypoxia (<2.5 mg/L) and mass endotoxin release.

Seasonal Lake Inversion & Toxic Sediment Overturn: Monitored via dual-depth temperature probes (0.5m vs 3.0m). Breakdown of thermal stratification brings anoxic, foul-smelling, pathogen-rich hypolimnion sediments to the surface.

Stormwater Flash First-Flush Runoff: Tipping-bucket rain gauge combined with water level sensor logs sudden rain head, alerting municipal gates to runoff carrying street fecal sludge and heavy metals into lake bays.

# 3. Quote 1: The Bare-Minimum 4-Sensor Sentry (₹35,500)

Designed for hackathons, early lab testing, and student grant proof-of-concept. It deploys the 4 essential surrogate sensors (EC + Turbidity + pH + Temperature) responsible for 85% of sewage detection capability, stripping all non-essential items.

| Subsystem | Component Description | Candidate Model | Interface | Qty | Cost (₹) |
| --- | --- | --- | --- | --- | --- |
| In-Situ Sensors | Industrial 4-Electrode EC / Salinity Sensor | Rika / OEM Graphite Modbus | RS-485 | 1 | ₹6,500 |
| In-Situ Sensors | Industrial Near-IR 860nm Optical Turbidity | OEM Optical Modbus (Static) | RS-485 | 1 | ₹9,500 |
| In-Situ Sensors | Industrial Flat-Glass Isolated pH Sensor | OEM Double-Junction Glass | RS-485 | 1 | ₹5,000 |
| In-Situ Sensors | Waterproof SS316 Temperature Probe | DS18B20 Encapsulated | 1-Wire | 1 pair | ₹600 |
| Edge Compute | ESP32 Microcontroller + MAX485 + Isolated DC-DC | ESP32-WROOM-32U + B0505S | GPIO/UART | 1 set | ₹1,500 |
| Edge Compute | SIMCom A7672S 4G LTE-Cat1 Cellular Module | A7672S Breakout + 3dBi Ant | UART | 1 | ₹2,200 |
| Solar Power | 20W Solar Panel + 10A PWM Charge Controller | Poly/Mono 20W + PWM 10A | DC Solar | 1 set | ₹2,200 |
| Solar Power | 12V 7Ah Sealed Lead-Acid Battery | Exide Powersafe 12V 7Ah | DC 12V | 1 | ₹1,500 |
| Enclosure & Float | IP65 ABS Weatherproof Junction Box + Glands | ABS 250x180x100 mm | IP65 | 1 | ₹1,200 |
| Enclosure & Float | DIY 4-Inch PVC Catamaran Pontoon Float + Anchor | Local PVC Pipe Assembly | Buoyancy | 1 set | ₹3,500 |
| Anti-Fouling | Perforated PVC Shroud with Copper Scrubber Mesh | Local Fabricated Shroud | Physical | 1 set | ₹800 |
| Calibration | NIST Traceable pH & EC Standard Calibration Sachets | Chemical Buffer Packets | Chemical | 1 pack | ₹1,000 |
| TOTAL QUOTE 1 INITIAL CapEx |  |  |  |  | ₹35,500 |

# 4. Quote 2: Value-Engineered Field Pilot (₹71,500) — Recommended Sweet Spot

The ideal configuration for university incubation (BITS Pilani, Rajasthan iStart) and 3 to 6-month continuous field validation in Fateh Sagar Lake. It adds Dissolved Oxygen (DO), water level, tipping rain gauge, and barometer while eliminating commercial branded markups by leveraging college laboratory synergies.

| Subsystem | Component Description | Candidate Model | Interface | Qty | Cost (₹) |
| --- | --- | --- | --- | --- | --- |
| In-Situ Sensors | Galvanic Dissolved Oxygen (DO) Probe + Modbus | DFRobot / OEM Galvanic DO | RS-485 | 1 | ₹14,000 |
| In-Situ Sensors | Industrial Optical Turbidity Sensor (860nm) | OEM Optical Modbus | RS-485 | 1 | ₹9,500 |
| In-Situ Sensors | Industrial 4-Electrode EC / Salinity Probe | Rika / OEM Graphite Modbus | RS-485 | 1 | ₹6,500 |
| In-Situ Sensors | Industrial Flat-Glass Isolated pH Sensor | OEM Double-Junction Glass | RS-485 | 1 | ₹5,000 |
| In-Situ Sensors | Dual-Depth Precision Water Temp Profiler | Paired SS316 DS18B20 | 1-Wire | 1 pair | ₹600 |
| Hydrology & Weather | Waterproof Ultrasonic / Hydrostatic Level Transducer | JSN-SR04T / WP-401B OEM | RS-485 / Pulse | 1 | ₹2,500 |
| Hydrology & Weather | Tipping Bucket Rain Gauge + BMP280 Barometer | Misol Tipping + BMP280 | Pulse / I2C | 1 set | ₹2,800 |
| Edge & Telemetry | ESP32-S3 Board + Isolated RS-485 Hub + RTC + SD | ESP32-S3 + ADM2483 + SD | Multi-UART | 1 set | ₹2,800 |
| Edge & Telemetry | SIMCom A7672S 4G LTE-Cat1 Cellular Modem + Antenna | Quectel / SIMCom 4G Module | UART | 1 | ₹2,400 |
| Edge & Telemetry | Station Power Board, Surge TVS & Isolated DC-DC | TVS + B0505S Converters | Internal | 1 set | ₹1,500 |
| Solar Power | 25W Monocrystalline Solar Panel + Aluminum Mount | Grade-A 25W Mono | MC4 Solar | 1 | ₹2,200 |
| Solar Power | 12V 12Ah Deep-Cycle Battery Pack + BMS | 12V 12Ah LiFePO4 / SLA | DC 12V | 1 | ₹4,500 |
| Solar Power | 10A MPPT / Smart Solar Charge Controller | Lumiao / EpEver 10A | Solar Control | 1 | ₹1,500 |
| Mechanical & Buoy | IP66 Weatherproof Polycarbonate Station Box | Fibox / Hensel IP66 | NEMA 4X | 1 | ₹2,500 |
| Mechanical & Buoy | Modular PVC/FRP Pontoon Float + Chain & Anchor | Catamaran Pontoon Frame | Buoyancy | 1 set | ₹5,500 |
| Mechanical & Buoy | Perforated Sensor Shroud + Copper Mesh Wrap | SS/PVC Cage + Copper | Anti-Fouling | 1 set | ₹2,200 |
| Mechanical & Buoy | Submersible Shielded Polyurethane Cabling (5m drops) | PUR Shielded Multicore | Submersible | 1 set | ₹2,500 |
| Validation Gear | Certified Calibration Standards Pack (pH, EC, DO) | NIST Buffer Solutions | Chemical | 1 pack | ₹3,000 |
| Validation Gear | College Laboratory Bridge (Reference Meter & Culture) | Techno NJR / MLSU Lab Loan | QA Synergy | Loaned | ₹0 |
| TOTAL QUOTE 2 INITIAL CapEx (Recommended Pilot) |  |  |  |  | ₹71,500 |

# 5. Quote 3: Municipal Full-Spec Enterprise Station (₹3,15,000)

The complete Smart City tender and municipal concession grade specification. Deploys autonomous motorized wipers on optical DO and Turbidity, 50W solar panel with 7-day monsoon LiFePO4 battery autonomy, heavy SS316 marine mooring buoy, handheld Apera validation meter, Aquagenx CBT field kits, and certified fortnightly NABL laboratory verification.

| Subsystem | Component Description | Candidate Model | Interface | Qty | Cost (₹) |
| --- | --- | --- | --- | --- | --- |
| Category A: Water Probes | Optical DO with motorized wiper | Yosemitech Y504-A | RS-485 Modbus | 1 | ₹48,000 |
| Category A: Water Probes | Industrial isolated pH sensor | Sensorex S8000 / Renke | RS-485 Modbus | 1 | ₹18,500 |
| Category A: Water Probes | 4-Electrode EC / Salinity sensor | Rika RK500-08 | RS-485 Modbus | 1 | ₹22,000 |
| Category A: Water Probes | Optical Turbidity with motorized wiper | Yosemitech Y510-A | RS-485 Modbus | 1 | ₹44,000 |
| Category A: Water Probes | Dual-depth SS316 Temp probes (0.5m & 3m) | PT1000 Class A RTD | Digital 1-Wire | 1 pair | ₹3,500 |
| Category A: Water Probes | Galvanic Isolation Hub & Sensor Modbus Board | ADM2483 / B0505S | Internal Bus | 1 set | ₹20,000 |
| Category B: Hydro & Weather | Submersible Hydrostatic Level Sensor (0-5m) | Rika RK500-01 | RS-485 Modbus | 1 | ₹14,000 |
| Category B: Hydro & Weather | Barometric Pressure Sensor in Radiation Shield | Bosch BMP280 | I2C / RS-485 | 1 | ₹1,800 |
| Category B: Hydro & Weather | Tipping Bucket Rain Gauge (0.2 mm resolution) | Misol WH-SP-RG | Pulse Reed | 1 | ₹5,500 |
| Category C: Edge & 4G | ESP32-S3 Industrial Controller Unit | ESP32-S3 Industrial | Multi-UART | 1 | ₹6,500 |
| Category C: Edge & 4G | Quectel 4G LTE-M / NB-IoT Modem + 5dBi Antenna | Quectel EC200U-CN | UART / USB | 1 | ₹5,200 |
| Category C: Edge & 4G | Surge Protection, RTC DS3231, SD Backup | Industrial TVS/RTC | Internal | 1 set | ₹4,300 |
| Category D: Solar Power | 50W Monocrystalline PV Panel + Bracket | Loom Solar / Waaree | MC4 DC | 1 | ₹3,500 |
| Category D: Solar Power | 12V 24Ah LiFePO4 Battery Pack + Smart BMS | Custom LiFePO4 Pack | DC 12V 24Ah | 1 | ₹14,500 |
| Category D: Solar Power | 10A MPPT Solar Charge Controller with RS-485 | Victron / Lumiao 10A | RS-485 Modbus | 1 | ₹4,200 |
| Category D: Solar Power | Marine Wiring, MC4 Connectors & Gland Plates | Marine Tinned Copper | Wiring | 1 set | ₹2,500 |
| Category E: Marine Mech | IP67 Polycarbonate Station Enclosure | Hensel / Fibox IP67 | NEMA 4X / IP67 | 1 | ₹5,500 |
| Category E: Marine Mech | Marine Anodized Aluminum Mast & Mounts | 6061-T6 Aluminum | Mechanical | 1 | ₹7,500 |
| Category E: Marine Mech | Perforated SS316 Sensor Cage + Copper Mesh | Marine SS316 + CuNi | Physical Shield | 1 | ₹8,500 |
| Category E: Marine Mech | Rotational Polyethylene Mooring Buoy + Anchor | UV-HDPE Buoy + Chain | Marine Mooring | 1 | ₹16,000 |
| Category E: Marine Mech | Submersible Shielded Polyurethane Cables (10m) | PUR Kevlar Jacket | Submersible | 1 set | ₹6,500 |
| Category F: QA/QC Gear | Handheld Multi-Parameter Reference Meter | Apera PC60-Z / HI98194 | Field Check | 1 | ₹32,000 |
| Category F: QA/QC Gear | NIST Traceable Buffer Solutions (pH, EC, DO) | Certified Standard Buffers | Chemical | 1 pack | ₹6,000 |
| Category F: QA/QC Gear | Aquagenx CBT E. coli Portable Field Kits | Aquagenx CBT (20 tests) | Field Bio-assay | 1 kit | ₹15,000 |
| TOTAL QUOTE 3 INITIAL CapEx (Municipal Full-Spec) |  |  |  |  | ₹3,15,000 |

# 6. Modular Upgrade Roadmap: Scaling from Quote 1 to Quote 3

A key engineering advantage of Aarogya Jal is its unified Modbus RTU / RS-485 architecture. Because all three quotes share the same industrial communication bus and ESP32 telemetry codebase, funding bodies do not suffer 'sunk capital' when upgrading:

Phase 1A (Student Hackathon / ₹35,500): Deploy Quote 1 at college jetty. Proves cloud telemetry, sewage conductivity spike alerts, and basic web dashboard.

Phase 1B (Incubator Grant / +₹36,000 incremental): Add Galvanic DO, ultrasonic level, and rain gauge to achieve Quote 2 capability. Conduct 3-month continuous lake monitoring.

Phase 1C (Municipal Concession / +₹2,40,000 incremental): Retrofit motorized wiper optical DO & Turbidity heads, upgrade to 50W LiFePO4 power system, deploy rotationally molded marine buoy, and onboard certified NABL bi-weekly testing.

Submitted by:
Manan Ramani & Team Aarogya Jal
Department of Computer Science & Engineering • Techno India NJR Institute of Technology, Udaipur
In Collaboration with Udaipur Municipal Corporation & Smart City Mission